[image: image1.jpg]




Ducati 1199 Superleggera 

ความมหัศจรรย์อันเป็นผลมาจากน้ำหนักที่เบาอย่างเหลือเชื่อ เพียง 155 กิโลกรัม ต่อพละกำลังมหาศาลกว่า 200 แรงม้าใน Ducati 1199 Superleggera  ด้วยพื้นฐานสุดยอดนวัตกรรมของ 1199 Panigale เสริมด้วยสมรรถนะของการเลือกสรรวัสดุที่พิเศษสุด เพื่อสรรค์สร้างผลงานที่เหนืออื่นใด ด้วยวัสดุเดียวกันกับวิศวกรรมอากาศยาน เช่น Titanium ไทเทเนียม, Tungsten ทังสเตน พร้อมระบบอิเลคทรอนิกส์และอุปกรณ์ชั้นนำ สำหรับสุดยอดยนตรกรรมแห่งทศวรรษที่จำกัดเอกสิทธิ์ไว้เพียง 500 คันทุกมุมโลก
[image: image2.jpg]



[image: image3.jpg]



Ducati 1199 Superleggera
Technical Specification
Engine

Type

Superquadro: L-twin cylinder, Desmodromic valve actuation, 4 valves per cylinder,  



liquid cooled
Displacement

1,198 cc
Bore x Stroke

112x60.8 mm
Compression ratio
13.2:1
Power


exceeds 200 hp (149 Kw) @ 11,500 rpm

Torque


134 Nm (99 ft-lbs) @ 10,200 rpm

Fuel injection
Mitsubishi electronic fuel injection system. Twin injectors per cylinder. Full Ride-by-Wire elliptical throttle bodies.
Exhaust

2-1-2 system, primary tubes in titanium. Twin titanium mufflers. 2 lambda probes
and 2 catalytic converters.
Transmission

Gearbox

6 speed 

Primary drive

Straight cut gears, Ratio 1.77:1
Ratio


1=37/15 2=30/16 3=27/18 4=25/20 5=24/22 6=23/24

Final drive

Chain; Front sprocket Z=15; Rear sprocket Z=41
Clutch


Hydraulically controlled slipper/self-servo wet multiplate clutch with Superbike 




configuration

Chassis

Frame


Monocoque in magnesium alloy 
Front suspension
Fully adjustable 43 mm USD Öhlins FL 916 fork with TiN treatment billet fork

bottoms and lightweight stress-optimized outer tubes 
Front wheel

3-spoke magnesium alloy 3.50" x 17"

Front Tyre

Pirelli Diablo Supercorsa SP 120/70 ZR17 
Rear suspension
Fully adjustable single Öhlins TTx36 shock  with titanium spring, with linkage

adjustable between progressive/flat. Aluminium single-sided swingarm. Adjustable

swingarm pivot.
Rear wheel

3 spoke magnesium alloy 6.00” x 17”

Rear tyre

Pirelli Diablo Supercorsa SP 200/55 ZR17 
wheel travel (front/rear)
120 mm (4.7 in) – 130 mm (5.1 in)

Front brake

2 x 330 mm semi-floating discs, radially mounted Brembo Monobloc Evo M50 




4-piston calipers with ABS, Brembo MCS 19-21 adjustable-ratio front brake master

cylinder, remote lever adjuster 

Rear brake

245 mm disc, 2-piston calliper with ABS 

Instrumentation

Colour TFT Display
Equipment

Riding Modes, Power Modes, ABS, DTC, DQS, EBC, DES, DDA+ w/GPS module, 




Fully RbW.DWC based on inertial platform. Dedicated buttons for rapid setting of 




DTC, DWC, EBC. Titanium rods. Carbon fibre parts: fairings, seat base, heel 




guards, air duct covers, power switch cover, rear shock cover, front and rear 




mudguards. Titanium bolts. 
Dimensions and Weight

Dry weight


155 kg (342 lb)
Kerb weight without fuel 
166 kg (366 lb)

Kerb weight


177 kg (390 lb)

Seat height


830 mm (32.7 in)

Fuel tank capacity

17 l (4.5 US gallons)

Number of seats

Single seat

Weight 
Reduction
Component


Weight Saved (vs. 1199 Panigale R)

Carbon fibre fairings and
-1.0 Kg (-2.2 lb)

assorted pieces

Magnesium Alloy 

-1.0 Kg (-2.2 lb) 

Monoscocca mainframe

Carbon fibre rear subframe
-1.2 Kg (-2.7 lb)

Titanium exhaust system
-2.5 Kg (-5.5 lb)

Forged and machined

-1.0 Kg (-2.2 lb)

magnesium wheels

Lithium-Ion battery

-2.0 Kg (-4.4 lb)

Custom front fork and rear 
-1.4 Kg (-3.1 lb)

shock

Superbike pistons

-194 g (-4 lb)

Crankshaft with tungsten
-396 g (-9 lb)

counterweights

SBK chain and rear sprocket, 
-1.3 Kg (-2.9 lb)

and various carbon fibre parts,

fasteners, and machined-from-

billet components

12 Kg (26.5 lb) overall (wet) weight savings vs. 1199 Panigale R
Warranty

Warranty (months)

2 years unlimited mileage

Maintenance (km/ months)
12,000 km (7,500 mi)/12 months

Valve clearance adjustment(km)
12,000 km (7,500 mi) – street use
Emissions 


Euro 3 

Ducatisti Co., Ltd.


12 Soi Vibhavadirangsit 22, Vibhavadirangsit  Rd.,



www.ducatithailand.com


Chomphon Chatuchak, Bangkok 10900








Tel: +66 (0) 2513-9999

Fax: +66 (0) 2513-7888


